For installing and configuration go to following address:

Installation and Use of the Visual Studio Code ESP-IDF Extension

https://www.youtube.com/watch?v=XDDcS7HQNlI

After configuration select COM port and your board on bottom menu of VScode like bellow images:

UART should be selected if you are using serial communication.

For creating new project from scratch first select a folder then select terminal and write:

idf.py create-project –p . TEST

idf.py create-project -p . app_main

But I don\t Know why I’m getting error

Solution:

https://www.esp32.com/viewtopic.php?t=32794&start=10

Hi Jorgen, I have the same problem, i.e. idf.py does not run in the PowerShell terminal which is opened by clicking Terminal and then New Terminal. However I get round this by opening an ESP-IDF terminal, this can be done by pressing ctrl E, releasing and then pressing t, or by pressing F1 to bring up the command pallet, type in ESP-IDF to filter out unwanted commands and then find Open ESD-IDF Terminal and click on it. Within the ESP-IDF terminal idf.py works perfectly. Note that I did install the environment variables by hand.

To include library watch:

# ESP32 - How to create your First ESP IDF project (From Scratch)

https://www.youtube.com/watch?v=oHHOCdmLiII&t=16s

Time:   8:14/13:10

How use configmenu:

Press F1 and type Terminal:

Select ESP-IDF Terminal

Now in Terminal write:

Start idf.py menuconfig

For example you can use it to change cpu frequency of ESP32

ESP-IDF Hint: esp_mac.h header file is not included by esp_system.h anymore. It shall then be manually included with #include "esp_mac.h"

Add :

"C:/Users/Iran/esp/**",

"C:/Espressif/**",

"C:/Users/Iran/esp/v5.2.2/esp-idf/components/log/include"

To Json file of your project

W (273) spi_flash: Detected size(8192k) larger than the size in the binary image header(2048k). Using the size in the binary image header.

Solution:

Solution: file CMkeList.txt should contains .c file name inside like image below

Bare Metal programing of ESP32

# Bare metal GPIO driver for ESP32 | Toggling an LED

#include <stdio.h>

//#include <cstdint>

#include "FreeRTOS.h"

#include "task.h"

#define GPIO_OUT_W1TS_REG 0x60004008

#define GPIO_OUT_W1TC_REG 0x6000400c

#define GPIO_ENABLE_REG   0x60004020

#define GPIO5             5

#define DELAY_MS          500

void app_main(void)

{

volatile __UINT32_TYPE__* gpio_out_w1ts_reg = (volatile __UINT32_TYPE__*) GPIO_OUT_W1TS_REG;

volatile __UINT32_TYPE__* gpio_out_w1tc_reg = (volatile __UINT32_TYPE__*) GPIO_OUT_W1TC_REG;

volatile __UINT32_TYPE__* gpio_enable_reg = (volatile __UINT32_TYPE__*) GPIO_ENABLE_REG;

*gpio_enable_reg = (1 << 5);

*gpio_out_w1ts_reg = (1 << 5);

*gpio_out_w1tc_reg = (1 << 5);

}

https://www.youtube.com/watch?v=QrJf6CF_g_8

identifier "CONFIG_FREERTOS_HZ" is undefined

Sometimes you change something in code but when it is built a deferent project although you just opened your project folder.

Solution:

You should full clean

I couldn’t use GPIO43 and GPIO44 as normal input and output. It just like the freeRTOS or bootloader uses this pins as cannel for console output:

Solution:

Default is UART0 and I put that on none  and add following codes to your code. And remember to use  to Full clean

//uart_driver_delete(UART_NUM_0);

gpio_reset_pin(GPIO_NUM_43);

gpio_reset_pin(GPIO_NUM_44);